KRC-8510 Balanced Air Inflation System

Operating Instructions:

- Connect to air supply, no greater than 80 PSI

- Connect tire inflation lines to the tires before turning valve on (1-4 air lines)

- Slowly turn on valve (if valve is opened too quickly, the inflation protection system will detect the surge in pressure and activate)

- If system activates before complete tire inflation, close supply valve and reopen slowly

- Tires will inflate to approximately 40lbs and inflation system will activate 40 PSI pop-off valve briefly once tires are up to pressure at approximately 40 PSI (this will be an audible, small burst of air) NO more air will inflate tire

- Pressure on gauge does not reflect actual tire pressure until tires have been fully inflated

- Checking tire pressure during inflation. Turn off supply valve and gauge will read actual tire pressure

- Air supply is sufficient to inflate 1 or 4 tires in the same time period

KRC-8510						       KRC-8512